Epilog Laser - Guide

Computer password is: Education

1. Design: Quick Overview

| Use adobe illustrator to create your design. | The software is available in the self-serve menu. |
| --- | --- |

In Adobe Illustrator...

| 1. FILE - NEW Select file and new, to create a document. Enter the size of your document. The maximum size is 18" wide by 12" high. |  |
| --- | --- |
| 2. VIEW ~ RULERS ~ SHOW RULERS Select VIEW~RULERS ~SHOW RULERS to show the rulers on the side of the screen. This is very helpful for precisely placing items in your design. |  |
| 3. FILE ~ PLACE To place an .jpg .pdf image/logo in your design, go to FILE ~PLACE. Tip - Click on the corner square of your image and hold the shift key if you wish to resize and keep the proportions. |  |
| 4. OBJECT ~ RASTERIZE To transform your object in black and white for rasterizing (engraving),in the menu go to OBJECT ~ RASTERIZE and select grayscale in the color model option. |  |
| 5. OBJECT ~ IMAGE TRACE You can also adjust the various tones of your image by using OBJECT ~ IMAGE TRACE or select from the top bar - this is useful for removing the grey areas from your image - To trace and cut inside an image do Image Trace and - EXPAND converts tracing results into paths that you can cut |  |

| 6. Adding a Shape - Contour line Adding a contour line around your object Select the tool from the menu to your right , If you press option key at the same time, it will change the shape. make your shape transparent by selecting , make your stroke color black and adjust the thickness of your stroke. *** if you want a vector cut (cut your wood right through) set your stroke width to .001. *** |  |
| --- | --- |
| 7. FILE ~PRINT On the menu select FILE ~ PRINT. In this menu, select default in the print preset, then select the placement at the top left corner. This is very important. This will change the print preset back to custom. Select Print. An error message will appear, this is normal. Press OK. |  |

2. Printing: Quick Overview

| Open the Epilog Application from the start menu or from the dock. |  |
| --- | --- |
| These are all of the past jobs. Scroll up to find your job. Double click on your job. The "Job Information Window" will appear. |  |
| Job Information Window Select Auto Focus ON (it is on by default). Click on the blue folder at the bottom left to select the material settings. |  |
| Select the material you will be printing on (aluminum, wood, glass, plastic, ...). Click on SAVE. The "Job Information Window" will appear, click SAVE. This will bring you back to the initial job menu. |  |
| With your job selected in blue, you can click on the PRINT PREVIEW icon. Here you can select COMBINED, RASTER, VECTOR, Here you can decide to print just the Raster (engrave) or just the Vector (cut). Click SAVE to make the changes. |  |

Now the most important steps ...

| Turn on the Epilog Laser - black switch just beside the power cord |  |
| --- | --- |
| Turn on the air compressor. If you don't do this step your project will probably catch on fire. |  |
| Turn on the air filter. Press the red power button and press the + to ensure that the filter is at the maximum speed. |  |
| Place your material on the print bed. Your material needs to be placed right up against the numbered ruler in the top left corner and over the small aluminum lip. |  |
| Only now you can press print Your job will appear on the EPILOG machine menu, press the green go button |  |

Voila !

Epic Fails & Trouble Shooting

| 1. If you notice that the laser head jams against the print bed ... - perhaps because your material is too tall or - perhaps you stopped a jab and it did not adjust properly | Open the bottom and move the belt to lower the tray. |
| --- | --- |
| 2. Your vector lines did not cut, even though you set them at .001 in Adobe Illustrator A) in print preview, you might have selected only raster - click on the vector to see if the vector lines appear. If your vector lines appear, reprint your job with only the vector lines B) If your vector lines are not there, then go back into Adobe Illustrator, open your file, delete all elements that are raster and keep only the vector lines, go though the printing process again selecting vector in the print preview. This should correct the problem. |  |
| 3. I had this issue. I noticed that my designs we printing oddly and that the laser seemed to be hitting the end of the track with a thump noise. I cleaned the optical strip very carefully and it fixed the problem. I also cleaned the lens and mirror. Follow instructions in the guide. |  |
| 3. If you are printing on an iPhone or iPad, measure carefully where you are printing on the device. Don't print on the camera lens. |  |
| 4. The computer that comes with the printer operates on Windows. In Adobe Illustrator, the fonts might not be the same. You might need to do a few adjustments or select a different font. |  |
| Mike Snider is the Print Boss, don't hesitate to ask him for help. Please share your tips with me and I can add them here. michel.emery@yesnet.yk.ca |  |

Tips for Printing Photos

Cleaning up pencil images

1. scan your picture or  take a good picture with your ipad/phone

Open it in Preview

in preview select the toolbox icon

a few options will appear,  select the triangle

you will be able to adjust the exposure and contrast

Here are 2 options for great results when working with photos

A.) Import your image in photos

and apply filters to transform your image

When your image has been imported in photos, select this tab

B) Open your image in preview. In preview you have this button  which opens up these options. You can adjust the contrast, cut the image in a shape...